Варіант 8

  - Translate into Ukrainian

THE CAMERA (3 частина)

Using the flash

The Polaroid 35mm AutoFocus camera has a built-in automatic flash that will fire when the green flash ready light is on and the shutter button is pressed. If the red flash-charging light is on, the shutter button can be pressed but there may not be enough light for a well-exposed picture. Wait for the green flash-ready light before

taking pictures. If you are outdoors in sunlight, you may wish to use the fill-flash feature that will fill in shadows on your subject taken outdoors in bright sun.

1. Lightly press and release the shutter button to charge the green flash-ready light.

2. Press and hold the fill-flash button (upper left on top of camera).

3. Press the shutter button to take the picture.

Note: The flash-ready light next to the viewfinder has been designed to shut off after about a minute if not used. To re-activate the flash, depress the shutter button lightly and release.

Flash tips

For proper flash exposure, position your subject at the following distances from the camera: 1,2–3 m (using 100 speed film) 1,2–3,6 m (using 200 speed film) 1,2–4,2 m (using 400 speed film) If taking vertical pictures, be sure the flash is at the top. Wait a few seconds after the flash-ready light comes on before taking a picture in poorly lit areas to insure maximum flash brightness.

2. Translate into English

В процессе работы с техническим феном:

При удалении старых лакокрасочных материалов работать шпателем с ручкой достаточной длины, в противном случае возможны ожоги кисти. Также, нужно следить за положением головы – избегайте вдыхания токсичных газов, независимо от применения респиратора.

- Не держать термопистолет вертикально поверхности земли, а только под углом, чтобы снизить влияние отражаемого потока горячего воздуха.

- При выполнении потолочных работ запрещается держать фен прямо над головой – не исключена возможность попадания частичек расплавленного материала.

- При несанкционированном отключении электроэнергии немедленно вынуть вилку сетевого шнура из розетки.

- Смену насадок производить только в термоустойчивых перчатках или после выключения фена и полного остывания снимаемой насадки.

- Запрещается использовать строительный фен не по назначению, например, нельзя его применять вместо бытового фена для волос.

Завершив работу

- Не прикасайтесь к обрабатываемым объектам или поверхностям сразу после их обработки.

- Упаковку фена производить только после полного остывания всех элементов инструмента.

  - Словниковий  диктант.